- Isolation

  - Sea-Tox flush selector valve (3-way, pressure neutral, 1mm min port dia, 15psi max differential)

    - Solenoid

    - Oil compensated

    - Multi Port valves

  - Bulkhead valves (Seawater, 500 psi, 1mm min port, small scale, low power, size <C- battery, high reliability, overpressure shut-off)

    - Direct lift solenoid valves

    - Custom Body valves

    - Axial flow thru valves

    - Turn valves

    - Multi Port valves

  - Cartridge Array valves (Seawater, 500 psi, 1mm min port, small scale, low power, size <AAA- battery, flush free valve)

    - Direct lift solenoid valves

    - Custom Body valves

    - Turn valves

    - Axial pull valves

    - Dual port single throw valve (eliminates output check valve)

  - Cartridge check valves (reliable seal, 500 psid, <5psi opening, any orientation, non-corrosive, small scale <AAA batter)

    - Spring loaded check

  - Sea pressure fittings (500psi, 1mm min thru, non corroding, vibration resistant, and reliable seal)

    - ¼-28 flat seals

      - Dibafit

      - Upchurch

- Filter

  - 25mm dia (or similar area) filtering media

  - Filters integrated in seal mount/chip/cartridge