G3 Environmental Sample Processor (ESP)

Analytical Module

DRAFT Requirements and Interface Specification DRAFT

Version 1.0

28 October 2011

Point of contact:

Chris Scholin, 831-775-1779, scholin@mbari.org

Jim Birch, 831-775-2062, jbirch@mbari.org

Collaborators:

Dierdre Meldrum, Deirdre.meldrum@asu.edu

Cody Youngbull, cody.youngbull@asu.edu

Funding is provided by the Monterey Bay Aquarium Research Institute (MBARI) from the Moore Foundation project “qPCR-based analytical module for the Environmental Sample Processor (ESP)” (grant #2728).  Grant is slated to run four years, concurrent with the calendar year.  Year one ends Dec 31, 2011.

Concurrence sheet

ESP Analytical Module Requirements Specification Version 1.0

Date: _____________________

Submitted by:

___________________________________

Approval:

___________________________________

Executive summary  (needs work)

This document is the central repository both for specifications of requirements and for interface-control information for the G3 Environmental Sample Processor (ESP) Analytical Module(s) being developed at ASU’s Institute of Biodesign. This project is funded by a 4 year grant by the Moore Foundation, running from Nov 8, 2010 until Dec 31, 2014. The main section is 2. Requirements specifications for the Analytical Module, which documents requirements for the module that ASU is developing for MBARI. This includes electrical, electronic, software, mechanical, thermal, and optical aspects. Another section is 4. Requirements for the ESP fluidics related to , which will help MBARI configure and operate the G3 ESP to use the module effectively.

Since this is the authoritative repository of requirements and interface information, any substantial changes to the document after release Version 1.0 need to be made through a change request and approval process.

Table of contents

Concurrence sheet	3

Executive summary	4

Table of contents	5

Table of figures	7

Table of tables	8

Table of revisions	9

1.	Introduction	10

1.1.	Document content	10

1.2.	ESP and Moore Foundation background	10

1.3.	Goals for Analytical module project	10

1.4.	Analytical module scope definition	11

2.	Requirements specifications for the Analytical Module	12

2.1.	Voltage	12

2.2.	Current	12

2.3.	Energy	12

2.4.	Data interface standard	12

2.5.	Message syntax	12

2.6.	Commands and Responses	12

2.7.	Optical data	13

2.8.	Thermal data	13

2.9.	Data modes	13

2.10.	Detection algorithm	13

2.11.	Size	13

2.12.	Mass	13

2.13.	Shock & Vibration	Error! Bookmark not defined.

2.14.	Orientation while running	13

2.15.	Reaction temperatures	13

2.16.	Heating rate	13

2.17.	Cooling rate	13

2.18.	Thermal environment	13

2.19.	Humidity	13

2.20.	Number of optical channels	13

2.21.	Optical wavelengths	13

2.22.	Reaction volume	13

2.23.	Tubing outer diameter	13

2.24.	Serial numbers	13

2.25.	Fan reliability	13

2.26.	Documentation	13

3.	Examples of Analytical module protocols and their implementation	14

3.1.	Introduction	14

3.2.	Real-time PCR	14

3.3.	Melting curve	Error! Bookmark not defined.

3.4.	Thermal calibration	14

4.	Requirements for the ESP fluidics related to Analytical Module	15

4.1.	Introduction	15

4.2.	Reagents	15

4.3.	Valves	15

4.4.	Pumps	15

4.5.	Tubing	15

4.6.	Fluidics protocols	15

Table of figures

Figure 1: Analytical Module and various interfaces.	Error! Bookmark not defined.

Figure 2: Transit vibration (MIL-STD-810F Fig. 514.5C-I).	Error! Bookmark not defined.

Figure 3: Functional shock (MIL-STD-810F Table 516.5-I and Fig. 516.5-8).	Error! Bookmark not defined.

Figure 4: Vibration minimum integrity (MIL-STD-810F Fig. 514.5C-17).	Error! Bookmark not defined.

Figure 5: fluidics diagram using 3 valves to achieve a choice of 2 sample sources, 2 master-mixes, 8 different multiplexed reactions, and DNA extraction.	Error! Bookmark not defined.

Table of tables

Table 1: Document revisions.	9

Table 2: Details of commands and responses.	Error! Bookmark not defined.

Table 3: Transit drop test (MIL-STD-810F Table 516.5-VI).	Error! Bookmark not defined.

Table of revisions

Table 1: Document revisions.

| Date | Version | Major changes |
| --- | --- | --- |
| 2011_10_31 | 0.6 | First draft begun |

# Introduction

## Document content

This document is the central repository for specifications of requirements and interface-control information for the G3 Environmental Sample Processor (ESP) Analytical Module. The main section is 2. Requirements specifications for the Analytical Module, which documents the module that ASU is developing for MBARI. Another section is 4. Requirements for the ESP fluidics related to , which will help MBARI configure and operate the ESP to use the module effectively. To force clarity and make changes easier to track, information should appear in either one section or the other, not both.

This document will evolve continually up to the preliminary design review (PDR). After that point, the plan is to have an approved version of the document that is only changed by agreement of both ASU and MBARI through documented change requests.

## ESP and Moore Foundation background

The Analytical Module is being developed by ASU for MBARI using funding from a four year grant from the Moore Foundation.  This grant was part of the Marine Microbiology Initiative, and was focused on supporting the miniaturization and enhancement of the ESP.  Funding would be used to increase the analytical throughput (reaction speed and total number of samples) by an order of magnitude to enable new knowledge of specific marine microbes and how they function in natural environments.

ASU is participating as a co-investigator on the project with responsibility for delivering Analytical Modules for the project.

## Goals for Analytical Module project

The overall objective is to develop an analytical module for oceanographic microbiology to function in MBARI’s third generation Environmental Sample Processor (3G ESP). This analytical module will be used in the ESP to perform in-situ, real-time genetic analyses. The plan includes demonstrating qPCR  and microarray processing in a prototype in XXXXXX.

The general and specific goals for the module that feed into its requirements include

- Have performance similar to bench-top machines: appropriate thermal range, fast heating and cooling rates, et cetera.

- Be usable in the 3G ESP: small size, low power, moderate energy per reaction, compatible electronic interface, compatible fluidic interface, readily programmable, et cetera.

As part of this project, MBARI will provide guidance to help ASU configure and operate the ESP to use the Analytical Module effectively.

## Analytical Module scope definition

The 3G ESP will be a host platform capable of mating with various “analytical modules”. In typical use, the Analytical Module will receive electrical power, electronic commands, and a fluid sample from the G3 ESP. The Analytical Module will execute real-time PCR thermal protocols, perform fluorescence measurements, and return optical results.

Will the module require active fluidic functions (pumps, valves, et cetera)?

From ASU Proposal:

We propose to develop and validate a quantitative Polymerase Chain Reaction (qPCR)-based

analytical module for the ESP to perform high throughput detection and quantification of gene

expression of marine microbial communities. The analytical module will perform quantitative

qPCR for both DNA and RNA samples. To improve the throughput of the assays, we will

fabricate microfluidic chips patterned with embedded microwells, or use droplet based emulsion

methods as reaction chambers. Successful development of this instrument will accomplish two

major technical advances. First, genomic and metagenomic information of marine microbes will

be utilized for probe selection and design for qPCR. The qPCR analysis will incorporate both the

“signature” DNA (16s or 23s rRNA for relevant prokaryotes and 18s rRNA for relevant

eukaryotes) and conserved sequences of selected functional genes, such as those related to

nitrogen fixation and photosynthesis. The instrument will thus be capable of assessing both

composition and productivity in microbial communities at high specificity and sensitivity.

Second, chip-based design and smaller volume qPCR reactions will allow more sensitive

detection and quantification of multiple genes in a cost effect format that meets the requirements

of the next generation ESP.

# Requirements specifications for the Analytical Module

## Voltage

Nominal 10 – 18  volts

Absolute Maximum 10 – 24V

## Current

## Maximum 2 A continuous

Inrush 3A upon contact closure

## Energy

Need to think about this one

## Power Switched

## Analytical module power will be under control of the ESP and may be turned on and off

## Data interface standard

Physical protocol 5 wire RS-232

Naming convention is from ESP perspective

TX           Transmit Data    ESP Drives

RX           Receive Data      AM Drives

HSO       Handshake out ESP Drives

HSI         Handshake in     AM Drives

SGnd     Signal Ground

Handshake function

HSO signals AM that ESP is able to receive

HSI signal ESP that AM is able to receive

Baud Rate 1200,9600,19200,115200

## Message syntax

## Commands and Responses

In this section, we outline the types of commands and responses.

### Types of commands

The Analytical Module will accept the following types of commands from the Host.

### Types of responses

The Analytical Module will provide the following types of responses to the Host.

### Details of commands and responses

## Optical data

## Thermal data

## Data modes

## Detection algorithm

## Size

## Mass

## Orientation while running

## Sample delivery

## Inter-sample contamination

What the module cleans between samples

## Waste

## Reagents

## Reaction temperatures

## Heating rate

## Cooling rate

## Thermal environment

## Humidity

## Number of optical channels

## Optical wavelengths

## Reaction volume

## Tubing outer diameter

## Serial numbers

## Fan reliability

## Documentation

# Examples of Analytical Module protocols and their implementation

## Introduction

The rest of this section contains examples of Analytical-Module-specific protocols and their implementation as related to qPCR.

## Real-time PCR

### Description in English

### Description in the Analytical Module’s programming language

## Thermal calibration

### Description in English

### Description in the Analytical Module’s programming language

# Requirements for the ESP fluidics related to Analytical Module

## Introduction

This is the place to put requirements for the ESP fluidics related to the Analytical Module that are not specifically addressed by specifying requirements for the module, above. There should be no redundant information here, meaning no information should appear both here and above.

## Reagents

## Valves

## Pumps

## Tubing

## Fluidics protocols