Discussion of Aug 12, 2010 ESP Drifter Engineering Test.

- The drifter was deployed first with the skirt and then without the skirt.

- Instrumentation on the buoy included a Crossbow NAV440 and a 12500 lbf full scale load cell at the top of the E/M cable.

- The Crossbow N1 message and the GPS GGA message were logged.

- Instrumentation in the ESP included a Microstrain InertiaLink

- Data was taken from all sensors at 10 Hz except the GPS GGA message which was at some default but less than 10 Hz rate.

- A waverider buoy was deployed and tethered to the Shana Rae.  The tether line remained slack although the waverider was towed when the SR had to maneuver closer to the drifter.

- The sequence of events was recorded in the drifter Excel log book and duplicated here.

- Data was processed in Matlab with the following scripts

  - splitFile.m – used to parse the single data file generated by the Labview VI (Drifter2010-Ver7.vi) into individual files for tension, xBow and iLink data.

  - procDataWS2.m and procDataNS2.m (WS – with skirt, NS – no skirt) were used to process the individual data files into time series and PSDs.  procDataXS2 used pWelch and procDataXS used psd methods.  A separate script PSD and PSDTest was used to verify that with the same arguments, psd and pWelch generated exactly the same data.

  - procDataAll2 read the data sets generated by procDataWS2 and procDataNS2 and plotted the results.

  - LoadAndProcWaveData (written by Andy Hamilton) was used to process the waverider data into data sets for with the skirt and no skirt time periods subtracting out the tow periods.

- Conclusions

  - The results seemed consistent with expectations.  Unfortunately, wave amplitudes for this (and the 2 other deployments) were low.

  - The peak in buoy and ESP acceleration PSDs was higher in frequency relative to the peak in the waverider heave PSD.  This seems reasonable given that accelerations should be ω2 times the heave amplitude which emphasizes the higher frequencies in acceleration relative to heave.

  - The plots (presented below) show a clear decrease in motion with the skirt on.  However, this can be explained by the increase in wave energy during the “without skirt” period shown in the waverider Heave PSD plot.  Unfortunately, the low sea state make a definitive conclusion about the effects of the skirt difficult.  However, based on the MI&T model results and recommendations, Andy and I agree it is reasonable to do the deployment with the skirt on.