DRIFTER 2011 Network Requirements

- General

  - The Drifter Network (DN) shall provide all the hardware and software required to provide bi-directional communication between the ESP shipboard network (ESN) and the ESP buoy network (EBN).

  - The DN shall operate in 2 modes: Periodic Status (PS) and Direct Connection (DC).

    - In the PS mode, only critical operational data (see below) must be transmitted from the EBN to the ESN.

    - The DC mode shall provide the same functionality as the existing direct network connection to the ESP

    - The DC mode shall provide a direct connection to each of the engineering sensors (see below)

  - Messages from the Shipboard ESP computer to the Shipboard buoy computer shall be in some really easy to use format

  - PS messages from the Shipboard ESP computer to the Shipboard buoy computer shall be provided in the same format and at the same rate during a DC session.

  - The entire drifter system shall support a mission duration of at least 5 days with a goal of 10 days being highly desirable.

- Periodic Status Mode

  - In the PS mode, critical data from each of the sensors defined below shall be provided at a rate selected by the user from 1 message per minute to 1 message per 10 minutes.

  - Engineering data

    - Buoy position: The standard NMEA GGA message

    - Buoy motion: The buoy accelerometer Acceleration, Angular Rate & Magnetometer Vector ((0xCB) message) shall be sampled 10 times per second and the minimum, maximum and mean values provided.

    - Cable tension: Cable tension shall be sampled 10 times per second and the minimum, maximum and mean values provided in each message.

    - ESP motion:  The ESP accelerometer Acceleration, Angular Rate & Magnetometer Vector ((0xCB) message) shall be sampled 10 times per second and the minimum, maximum and mean values provided.

    - PAR Sensor

    - ADCP

  - ESP data

- Direct Connection

  - The DC mode shall provide the ability to interact from the ESN directly with each engineering sensor using the vendor’s standard message protocol.

  - The DC mode shall provide the ability to interact from the ESN directly with the ESP…