ESP Drifter Deployment Plan

Gene Massion

Revision

- May 12, 2011	Initial Draft

Ship: University of Hawaii R/V Kilo Moana

Departure Date: September 6, 2011

Return Date:	September 21, 2011

MBARI Crew

- Julie Robidart 		jrobidar@ucsc.edu

- Gene Massion		magene@mbari.org	831 7751922 (work)	831 4199188 (cell)

- Roman Marin		maro@mbari.org

Time Line (details TBD)

- September ( August?) x	MBARI Shipping Container arrives at UH Marine Center

  - UH offloads container and connects to shore power

- September (August?) x	MBARI crew arrives at UH Marine Center

- September x	MBARI crew moves ESP to 10’ container (Question 1,2)

- September x-y	MBARI crew prep and check ESP

- September x-y	MBARI & KM crew mount antennas and run cables

- September x	Load ship

- September x	Deploy ESP

- September x	Recover ESP

Deployment plan

Crew required: Deck supervisor, 2 tag line handlers, 1 quick release tender, 1 E/M cable tender, winch operator, crane operator, acoustic release operator

- Test Benthos release on winch wire at 23 meter depth

- Stage the ESP under the U-frame

  - Soft line on winch (Q4)

  - Mark the soft line at about 20 meters

  - MBARI supplied Benthos acoustic release between winch line and ESP pad eye (Q3)

  - 2 tag lines

  - E/M cable tender tends E/M cable between buoy and ESP

  - Check everything is clear for deployment

- Stage the buoy under the crane (port side of U-Frame?)

  - Crane hook to 20’ sling to MBARI supplied “Sea-catch” manual quick release to buoy bale REMOVE THE SAFETY PIN

  - 2 tag lines

  - Check the recovery line bundle and grapple loop

  - Check everything is clear for deployment

  - Quick release tender tends release line

- Confirm ESP and Buoy are working properly

- Put the buoy in the water slightly port of the U-frame and leave it on the crane hook

  - Put enough slack in the 20’ sling so the buoy isn’t snapping tight but the crane hook isn’t banging into the buoy

  - Recover the tag lines

- Put the ESP in water and lower away

  - Recover the tag lines

  - Slow down when approaching the 20 meter mark

  - We should see the ESP and the buoy start to move toward each other

  - Continue lowering until the winch line gets near the buoy

- Trigger the acoustic release

- Confirm ESP is free: should see the winch line swing away

- Recover winch line

- Pull the buoy quick release

- Done

Recovery Plan

Crew required: Deck supervisor, 2 tag line handlers, 1 E/M cable tender, winch operator, crane operator

- Find the buoy

  - We should have a recent GPS fix, a nav light on the buoy and a RF beacon (Q7)

- Position the KM as appropriate for grappling

- Grapple the recovery lines bundle off the buoy

  - The recovery lines bundle has a light line and loop for grappling.  The bundle itself will have a line attached to the buoy bale, a pair of tag lines for the buoy and the ESP safety/recovery line.  The ESP safety/recovery line is attached to a quick release on the bottom of the buoy.

  - The ESP safety/recovery line should release from the Buoy when we pull up the recovery lines bundle, if it doesn’t see below.

- Put the buoy recovery line on the crane hook (Q5)

  - Position the ESP relative to the U-frame similar to deployment

- Put the ESP recovery line on the winch wire/line

  - Note: if the ESP recovery line doesn’t release from the ESP when the recovery line bundle is grappled up from the buoy, skip step 5-6 and do the following

- Winch up the ESP to ~10 feet below the surface

- Recover the buoy

If the ESP recovery line didn’t release in step 5 do the following

  - Pick the buoy straight up until the buoy deck is a couple of feet higher than the KM deck

  - Swing the crane until the buoy is near the transom

  - Pull the ESP safety/recovery line release with a boat hook

  - Recover the buoy and attach the ESP recovery line to the winch and take the load on the winch (Q6)

- Recover the ESP

  - Attach tag lines ASAP with MBARI tag line boat hooks.

- Done

Questions

- Does UH already have a suitable refrigerated container?

- Is a fork lift available? Can qualified MBARI crew drive it?

- Answered- Can UH supply 2 acoustic releases (1 primary and 1 spare)?  Does the deckbox use a hull mounted transducer or a dunker?

- Does UH have appropriate soft line for the ESP (2500 lbf weight in air, ~20,000 lbf break strength)?  We don’t need a lot of line, can we wrap it over the top of the wire on the winch if necessary?

- Can the crane pick the buoy if it has a ~20’ recovery line?

- Can we pass a master link & shackles through the various turning blocks on the U-frame?  We have a wide mouth Skookum block if necessary.

- Answered- What kind of RF beacon locator does the KM have?  Will it work with a standard Novatech beacon?

UH Contact Information

Matt Church	Co-chief Scientist		mjchurch@hawaii.edu

Dan Fitzgerald	Ocean Technology Group Lead	danfitz@hawaii.edu

Blake Watkins					blakewat@hawaii.edu

Steve Poulos					poulos@soest.hawaii.edu

Ross Barnes 	Port Operations Manager 	(808) 842-9815 begin_of_the_skype_highlighting             end_of_the_skype_highlighting

Shipping Information:

Attn: Blake Watkins / (CMORE cruise Sept. 6-21, 2011 R/V Kilo Moana)

C/O UH Marine Center

#1 Sand Island Access Road

Honolulu, HI 96819

Phone (808) 842-9813 begin_of_the_skype_highlightingend_of_the_skype_highlighting, Fax (808) 842-9833 Toll Free 1-888-800-0460 begin_of_the_skype_highlighting