|  | Simulation of Lid2 Date: Friday, August 25, 2023 Designer: Solidworks Study name: Static 1 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 4 Connector Definitions 5 Interaction Information 5 Mesh information 5 Sensor Details 6 Resultant Forces 6 Beams 6 Study Results 7 Conclusion 9. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: Lid2 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Split Line1; Solid Body; Mass:2.53139 lb Volume:25.9514 in^3 Density:0.0975437 lb/in^3 Weight:2.52968 lbf; C:\Users\proberts\Desktop\kiwi_FEA model\Lid2.SLDPRT Aug 25 12:45:08 2023.

# Study Properties Study name; Static 1. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\proberts\Desktop\kiwi_FEA model).

# Units Unit system:; English (IPS). Length/Displacement; in. Temperature; Fahrenheit. Angular velocity; Hertz. Pressure/Stress; psi.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 39,885.4 psi. Tensile strength:; 44,961.7 psi. Elastic modulus:; 1.00076e+07 psi. Poisson's ratio:; 0.33. Mass density:; 0.0975437 lb/in^3. Shear modulus:; 3.77098e+06 psi. Thermal expansion coefficient:; 1.33333e-05 /Fahrenheit.; SolidBody 1(Split Line1)(Lid2). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. On Flat Faces-1; Entities:; 1 face(s). Type:; On Flat Faces. Translation:; 0.5, 0.5, 0. Units:; mm.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(lbf); -0.209513; 77,897.7; 0.358517; 77,897.7. Reaction Moment(lbf.in); 0; 0; 0; 0.. Load name; Load Image; Load Details. Pressure-1; Entities:; 22 face(s). Type:; Normal to selected face. Value:; 2,250. Units:; psi. Phase Angle:; 0. Units:; deg..

# Connector Definitions No Data

# Interaction Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Blended curvature-based mesh. Jacobian points for High quality mesh; 16 Points. Maximum element size; 0.284553 in. Minimum element size; 0.0210476 in. Mesh Quality; High. ## Mesh information - Details Total Nodes; 114568. Total Elements; 73113. Maximum Aspect Ratio; 5.6447. % of elements with Aspect Ratio < 3; 99.2. Percentage of elements with Aspect Ratio > 10; 0. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:07. Computer name:; TENAYA.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf; -0.209513; 77,897.7; 0.358517; 77,897.7. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf.in; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf; -0.00198143; 0.0152145; 0.00268057; 0.0155753. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf.in; 0; 0; 0; 8.85075e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 2.244e+01psi Node: 87340; 2.138e+04psi Node: 70264. Lid2-Static 1-Stress-Stress1. Name; Type; Min; Max. Displacement1; UY: Y Displacement; -2.821e-03in Node: 15802; 1.823e-04in Node: 2089. Lid2-Static 1-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 1.452e-06 Element: 49724; 1.801e-03 Element: 63348. Lid2-Static 1-Strain-Strain1. Name; Type; Min; Max. Stress2; VON: von Mises Stress; 2.244e+01psi Node: 87340; 2.138e+04psi Node: 70264. Lid2-Static 1-Stress-Stress2. Name; Type; Min; Max. Factor of Safety1; Automatic; 1.866e+00 Node: 70264; 3.000e+00 Node: 1. Lid2-Static 1-Factor of Safety-Factor of Safety1.

# Conclusion