R /V Paragon

Operation Checklist

Start Up

- Verify pre-cruise has been submitted

- Identify Shore Contact, fill out Float Plan & deliver to manifest box

- Fill out visitor waivers and deposit in manifest box if required

- Fill out Cruise Log

- Determine if fuel is sufficient for trip (dip if any uncertainty of level)

- Engine Power PERKO- Switch ON

- House Power PERKO- Switch ON

- Check high water alarm

- Check fuel level in gas hydraulics motor and refill if low

- Leave aft hatch open or run blowers for 3 minutes prior to starting engines

- Lower engines

- Center and secure boom

- Tie down and secure loads

- Secure anchor, chain and winch

- Turn on main breaker panel

- Set one VHF Radio to channel 16

- Turn on Garmin displays

- Brief new passengers on vessel safety

  - Hazard of falling overboard

  - Man overboard procedure – ring, sling, throw bag, and cargo net

  - Whereabouts of PFDs and survival suits

  - Requirements for work vests, hardhats, and steel-toed shoes

  - Watch head on boom capstan and T-bit

  - Radar – make sure it is off before going forward

  - Emergency communication on VHF 16, Garmin DSC, SPOT, cell phones and EPIRB

  - Fire extinguishers – location and procedure

  - First Aid and AED locations

  - Location of Flares

  - Abandon ship – Mayday call, Don immersion suit, take EPIRB and ditch bag, stay with vessel as long as possible, stay together

- Confirm that everyone on deck is wearing work vests

- Cast off once crew is ready

Underway

- Confirm lines and buoys are inboard and secured

- Confirm passengers on back deck are wearing proper safety gear

- Listen for high water alarm

- Pump out holds underway

- Turn on navigation lights in restricted visibility

- Turn on radar in restricted visibility

- Confirm VHF radio is set to channel 16

- Confirm boom is centered and secured

- Monitor blind spot for hazards

- Monitor engine diagnostics

- Always ask whether the crew and passengers are ready before accelerating

Shut Down

- Fuel boat if under 100 gallons on board

- Notify shore contact of your safe return

- Complete Cruise log

- Complete Fuel log

- Complete Maintenance log

- Add Ring Free Plus to new fuel

- Rinse Engines through cowling when vertical for 30 seconds each

- Raise Engines

- Rinse exterior of engines, bracket and tie-bar thoroughly

- Gravity flush engines 5 minutes apiece – DO NOT START ENGINE

- Replace rinse-hose fittings on engine

- Rinse boat thoroughly including windows, deck, and anchor winch

- Empty trash

- Stow work vests and hardhats NEATLY in V-berth

- Close and lock windows

- Tidy up the V-berth and cabin

- Clean Garmin monitors if smudges – ONLY use the cleaning kit stored behind the helm seat

- Replace covers on GPS and engine display

- Turn Engine Power PERKO Switch OFF

- Turn House Power PERKO Switch OFF

- Turn inverter OFF

- Turn Navigation lights- OFF

- Flip “AFT Bilge Pump” to Auto.  “Forward Bilge pump “ stays on - OFF

- Lower dive flags if still up (shouldn’t be up when underway)

- Contribute to general upkeep:

  - Wax windows

  - Wax engine cowlings (3M wax and towel)

  - Dab rust stains on fiberglass with gel

  - Scrub decks with boat soap and brush

  - Sweep and scrub cabin floor with soapy water and wet/dry shop vac

  - Vacuum v-berth

  - Clean monitors with proper dust free towels and cleaner

  - Oil teak rails

  - Remove dirt and mildew from life ring and fenders

- Confirm all tools are clean and returned

- Rinse lines and hang on the rail aft of the helm

- Rinse and bundle ratchet straps.  Stow in green bin in cabin.

- Untangle and replace the electric winch controller beneath the helm seat

- Center and secure the boom.  Add half hitch on a bight to both side-lines.

- Report any problems or maintenance requests

- Replace bolts to empty holes in mid-deck tie down rail

- Lock cabin and aft hatch with combo locks oriented right-side-up

- Stow and depressurize the dock hose - run the hose down and back on the Flyer dock

- Confirm the boat is secured to dock properly (Fenders in place and lines secure to cleats)

- Confirm the lines aren’t sitting on the electric fence.

- Finish lines with a Flemish Flake (yes please.  It is quite nice)

- Plug in sealion electric fence on Flyer dock.

- Replace the caution sign at top of ramp on exit.

Fueling

- Dip fuel tanks and note volume of each tank in cruise log

- Insert fuel nozzle and make contact with the fill port to discharge static

- Fuel with 89 Octane gas to a volume of 100 gallons in the saddle tanks

- Do NOT leave the nozzle while filling

- Add Ring Free Plus to fuel (1 oz/10 gallons)

- Fill out Cruise Log

- Fill out Fuel Log