OCEAN ENGINEERING DEPARTMENT

ELECTRICAL FABRICATION WORK REQUEST

| Originator | Brenton Salmi |
| --- | --- |
| Project | MARS2 |
| Job Description | Rework board to final version |
| Submitted Date | 10/17/2024 |
| Estimated Hours | 2 |
| Desired Completion Date | 10/24/2024 |
| Worktag | AC103988 |

Drawing file paths and names:

- HPB Altium 365 Page:

  - https://applied-physics-lab.365.altium.com/designs/8B49F1D1-BD12-425D-A8D5-7BF6C8A4ACE9?variant=MAIN&activeDocumentId=Coversheet.SchDoc(1)&activeView=SCH&location=[1,98.47,15.41,19.94]#design

# Board States

| Board SN | State | Needed Rework |
| --- | --- | --- |
| 1 | Almost Completed | Install J18, J17, J7, J9, J12 |
| 2 | Completed | Just clean PCBA |
| 3 | Not Reworked | Perform all rework steps |

# Work Description

## High Level

- Components will be DNP’d

- Components will be installed

## DNP Component(s)

| REF DES | QTY Total |
| --- | --- |
| R12 | 1 |
| R13 | 1 |
| R43 | 1 |
| R47 | 1 |

## Install Component(s)

When installing the Harwin connectors please install using Loctite 271 prior to soldering.

| REF DES | Part Number | QTY Total |
| --- | --- | --- |
| R21, R25 | ERJ-3EKF4992V | 2 |
| J3, J4 | M80-5000000M5-02-333-00-000 | 2 |
| J5 | M80-5400642 | 1 |
| J6, J7, J8, J9, J11, J12, J13, J14, J15, J16, J17, J18 | M80-5400442 | 12 |
| J10 | M80-5401442 | 1 |

## Clean the PCB

Clean the entire PCB using IPA.