During final assembly 2Mohm bleeder resistors were added across the 375V capacitors to provide a slow discharge for safety.

MFG: Vishay

PN: VR37000002004JR500

These are through-hole 0.5W metal film resistors.

At 375V they are dissipating ~70mW (of 500mW rating).

The bulk capacitors are:

- MFG: Kyocera AVX

- PN: FFVE6B0147KJE

- 140uF 800V

They are expected to discharge at roughly the following rate using the t=RC time constant of ~280 seconds.

- 1RC (4.6 minutes)

  - 138V

- 2RC (9.3 minutes)

  - 50.6V

- 3RC (14 minutes)

  - 18.75V

- 4RC (18.6 minutes)

  - 6.9V

- 5RC (23.3 minutes)

  - 2.25V

This resistor is placed directly across the capacitors leads.