MARS2 Functional Requirements - PRELIMINARY

Version 0	Initial draft		T. McGinnis	21 May 2020

Version1.0	Edit 1			C. Dawe	22 May 2020

- Shore Station

  - Power

    - -1500VDC, 3kW

    - Power return to Shore using seawater

  - Data

    - 2 Fibers – Gigabit Ethernet over Fiber to J-Box Network Switch planned on using 4 similar to WHOI plan 2 probably is good

    - 1 or 2 Fibers – J-Box Controller Console Portplanned on using the switches above to create primary path to controller

    - 1 or 2 Fibers – J-Box Network Switch Console Port single fiber to ethernet switch (4 port x 2 SFPs + 2 CU to 4 port unmanaged Ethernet switch

- Seafloor Cable Interface (MARS Shore Station)

  - Input power

    - -1500VDC, 3kW

    - 80-90% efficiency, typical

  - Power control & monitoring

    - ?? currently have a PC104 stack connected to the control subnet providing V,I status, no control, it is all done manually through an MBARI built control box (may not work with the Schaefer PS)

  - Optical Fibers

    - Pass-through

- Junction Box

  - J-Box Subsystem Level Monitoring

    - Input voltage and current

    - Hotel power voltage and current

    - Temperature

    - Humidity/water intrusion

    - Tilt/attitude (?)

  - J-Box Subsystem Level Controls

    - ??

  - Sensor Ports

    - Quantity

      - 8

    - Power

      - 375VDC/5A All

      - 48VDC/4A Ports 1,2,3,4,6,7,8

    - Data

      - 10/100BaseT Ports 1,2,3,4,6,7,8, Port 5 will be gigE from a media converter Port 5 will also have two of the spare fibers from the sea cable passed through

    - Timing

      - 1PPS, using IEEE1588 (?) Meinberg M600 Master and Meinberg Syncbox N2x slave

    - Connector Type

      - ODI Nautilus 12-Way Electrical Port 5 will be a hybrid EO connector with 375V pin 1 375C Pin 2 and 4 fibers 2 passed through from the sea cable, one connected to a media converter (Cu to FO) from the switches subnet 10.1.15.xxx

    - Connector Pin Assignment

Pin 1      reserved

Pin 2      reserved

Pin 3      Ethernet Tx+

Pin 4      Ethernet Tx-

Pin 5      1PPS+

Pin 6      1PPS-

Pin 7      Ethernet Rx+

Pin 8      Ethernet Rx-

Pin 9      375V (+187VDC, relative to seawater?) I know in the current design these are referenced this way, why?

Pin 10    375C (-187VDC, relative to seawater?)

Pin 11    48V

Pin 12    48C

    - Sensor Port Control

      - Hotel load power switching

      - Instrument power switching for 48VDC and 375VDC

      - Power conductor mechanical relay isolation switching

      - User settable port current limit for 48VDC and 375VDC power

    - Sensor Port Monitoring

      - Power supply current for 48VDC and 375VDC

      - Ground Fault monitoring for 48VDC and 375VDC