# MARS In-band Backdoor

# General

MARS has an Xbar subsea to allow connection to serial ports on the Cisco’s, EDFA’s and power control.  The Xbar also has one port set for loopback.

# Accessing

The Xbar must be set to the appropriate port before serial port access can be done.  The port can be set by either of two ways.

Method 1 – use webpage to select port

Use a browser to connect to 10.1.128.20/cgi-bin/obc_xbar.

Select the desired port and then connect to 10.1.128.20:7001.

Method 2 – Directly specify port

The port can be directly selected of the desired port number is known.  Simply use a browser to connect to 10.1.128.20/cgi-bin/obc_xbar?port=x

Where x is one of the following:

- N/C

- N/C

- CiscoA

- CiscoB

- East EDFA

- West EDFA

- Power Controller

- Loopback