Phobos Deployment on MARS – Rev 11

Changes in red

General:

Test Tank Dates:  Week of January 19, 2014

Scheduled deployment date:  January 31, 2014

Vessel:	  Rachel Carson

Deployment method:  Float pack

Weight in air:  420 lbs OLD VALUE

Weight In water:  185 lbs OLD VALUE

Cable:  35 meter to SIIM to 5 meter oil filled hose with ODI connector (90 degree pen)

Port:  4

Networking:

Subsurface vlan: 	10.1.14.xxx

Subsurface gateway:	10.1.14.1

MARS O&M reserves address 10.1.14.250 to 10.1.14.255

Only addresses in the range of 10.1.14.2 to 10.1.14.11 can be mapped to external address

Four subsea addresses required (10.1.14.7 added since last deployment)

(10.91.44.7 was deleted)

Main Controller			10.1.14.2	134.89.42.142

Perle Server for camera control	10.1.14.3	134.89.42.143

DigiServer for CTD and ADCP	10.1.14.4	134.89.42.144

HD Video Feed			10.1.14.6	134.89.42.146

Light Control			10.1.14.7	134.89.42.147

Science vlan:		10.91.44.xxx

Science gateway:	10.91.44.1

MARS O&M reserves address 10.91.44.250 to 10.91.44.255

Only addresses in the range of 10.91.44.2 to 10.91.44.11 can be mapped to external address

Two surface addresses required.

Control computer			10.91.44.5	134.89.42.145

DNS Server #1:	10.91.128.61

NTP Server #1:	10.91.128.55

External mapping address range will be 134.89.42.140 to 134.89.42.149.

A maximum of 10 addresses can be mapped externally.

MARS O&M needs a list of computers by IP address that may access the externally mapped addresses.  (Maximum of 6)

Note:  For WNS testing, the external address range is 134.89.52.140 to 134.89.52.149

Power Requirement:

48 volts - none

375 volts – 1 amp max

Inrush current – 15 amps maximum, 3 ms duration.