Phobos Deployment on MARS – Rev 8

Changes in red

General:

Test Tank Dates:  Week of July 8, 2013

Scheduled deployment date:  July 25, 2013

Vessel:	  Rachel Carson

Deployment method:  Float pack

Weight in air:

Weight In water:

Cable:  70 meter to SIIM to 5 meter oil filled hose with ODI connector (90 degree pen)

Port:  4

Networking:

Subsurface vlan: 	10.1.14.xxx

Subsurface gateway:	10.1.14.1

MARS O&M reserves address 10.1.14.250 to 10.1.14.255

Only addresses in the range of 10.1.14.2 to 10.1.14.11 can be mapped to external address

Four subsea addresses required (10.1.14.6 added since last deployment)

Main Controller			10.1.14.2	134.89.42.142

Perle Server for camera control	10.1.14.3	134.89.42.143

DigiServer for CTD and ADCP	10.1.14.4	134.89.42.144

HD Video Feed			10.1.14.6	134.89.42.146

Science vlan:		10.91.44.xxx

Science gateway:	10.91.44.1

MARS O&M reserves address 10.91.44.250 to 10.91.44.255

Only addresses in the range of 10.91.44.2 to 10.91.44.11 can be mapped to external address

One surface address required.

Control computer			10.91.44.5	134.89.42.145

DNS Server #1:	10.91.128.61

DNS Server #2:	10.91.128.62

NTP Server #1:	10.91.128.55

External mapping address range will be 134.89.42.140 to 134.89.42.149.

A maximum of 10 addresses can be mapped externally.

MARS O&M needs a list of computers by IP address that may access the externally mapped addresses.  (Maximum of 6)

Note:  For WNS testing, the external address range is 134.89.52.140 to 134.89.52.149

Power Requirement:

48 volts - none

375 volts – 1 amp max

Inrush current – 15 amps maximum, 3 ms duration.