Microwave System Set Up Notes

8/2015

These settings are required for video and mooring downloads.

- Toro HP Ethernet switch:

  - IGME snooping enable

  - Admin mode enable

  - Filter unknown multicast DISABLE

- Toro Netgear Ethernet switch

  - IGME snooping enable

  - Admin mode enable

  - Filter unknown multicast ENABLE

- Moss Landing Ethernet switch:

  - IGME snooping enable

  - Admin mode enable

  - Filter unknown multicast DISABLE

- Toro Event HD6500 Ethernet payload ports

  - Auto negotiation disable

  - 100mb, Full, Symmetric

- Harris 18ghz Moss Landing Ethernet payload port

  - Install dumb switch between system and MBARI switch

  - 100m, Full