Vertical Microstructure Profiler (VMP) Casts Procedures

by Magdalena Carranza (MBARI)

Jun 20, 2022

VMP-200 Pre-Deployment Handling:

VMP-200 Post-Deployment Handling:

VMP-200 Data Download Procedure:

Use the Windows laptop in the pink case, which has Motocross, RSI Link and Zissou Essentials installed.

- Ensure the VMP is ready for data download.

- Connecting to the VMP:

  - Remove the deployment connector plug on the tail end and sure there is no residual water in the connector housing

  - Connect the charging/communications cable to the VMP

  - Plug the USB connectors into the laptop.

- Establishing the connection

  - Open the Terminal Emulator (MotoCross) and connect to USB – COM6 and Baud rate 9600.

  - Power the VMP (using magnet screw towards the head side of the VMP, i.e. near the probes).

  - Terminal should report power and start upp.

  - When the countdown to data acquisition begins, press “q” to abort data acquisition.

  - Type “usbl” in the prompt and press “enter/return”

- Downloading the data:

  - Open RSI Link (Rockland’s Propietary software)

  - Establish connection to USB by selecting the right COM port (COMM5):

  - Press the “Get Root Directory” button, and the window will show the directory structure in the CF card (in the VMP)>

  - Click on “DATA” directory

  - Select .P file(s) of interest and press “Download”

  - Select the folder in which to download the file(s) and press “Open”. (A dialogue box will open with the download in progress..)

  - Check that the file size in the download directory will match the file size in RSILink.

  - Copy file of interest from “Downloads” folder into:

/Destop/TurbulenceData/VMP200/DielCruise/

And fill in the VMP200.xls sheet

- Disconnect the VMP and charge:

  - Close RSILink

  - In the Terminal Emulator press “q”

  - Terminal will report the start-up

  - When countdown to data acquisition begins, press “q” to interrupt

  - Close terminal emulator

  - Disconnect USB connections

  - REMOVE POWER switch from VMP (i.e. magnetic screw)

  - Connect the Charging/Comms cable to the battery charger.