OASIS5 Basic Features

6/5/14

BRS and MRS controller spare. OASIS3 replacement, OA mooring replacement, Waveglider PCO2 system controller and general purpose instrument controller.

Features:

- PIC24/33/DSPIC processor

- SD card mass storage

- 512K-1MB RAM

- 32k xtal for RTC

- 8-20Mhz operation

- 2-3 SPI bus

- 12 instrument ports (DC+serial)

- 1 232 console port in addition to inst ports

- 3 instrument ports are switchable to 485 for Modbus expansion. Dip sw selectable.

- Relay isolation

- GF detection

- bat volts, can temp, can press, bat amps

- Photo cell LED enable

- WDT, internal to PIC

- External reset button (long reset)

- Micropower 3.3 regulator

- Coin cell for RTC

- USB-OTG

- Diode isolation for multi batteries

- Power loss/ shutdown detect circuit

Removed from O3 or O4:

- Wakeup bits

- WDT chip

- Separate  gnd pin on instrument ports (5 pins to 4)

- Extra A/D channels

- Expansion Boards

- I/O Board select lines

- GF daughter board

- A/D daughter board

Software Features:

- COTS RTOS

- Runs with existing OASIS4 app + interface layer.

- SD card support with FAT32 file system, long file names

- USB console and drive mounting

- Battery monitoring to replace Emeter

- Modbus serial port expansion on 3 485 ports

- Use PIC24 internal wdt

- Keep wake on char