IO Connections

SeeTrack CoPilot

Commercial in Confidence

Feb 21st 2013

- Table of Contents

1	SeeByte	4

1.1	Points of Contact	4

1.2	Awareness Made Easy	4

1.3	Commitment	4

2	Hardware Specification	4

2.1	Front Panel Layout	4

2.2	Rear Panel Layout	5

2.3	Interface box connections	7

2.4	Interface Box Front Panel Layout	8

2.5	Interface Box Rear Panel Layout	8

2.6	DB25 connector wiring layout	9

2.7	Wiring Diagram	10

Distribution: CUSTOMER

Revision History

| Revision | Date | Comment |
| --- | --- | --- |
| 01 | 21 Feb 2013 | New Template |

Authors: SeeTrack CoPilot Team

This document is commercial in confidence. If you have received it in error, please contact:

SeeByte

Registered in Scotland Number: SC194014

Registered Office: Orchard Brae House, Edinburgh, EH4 2HS

VAT Registration Number: 783656681

Tel: +44 (0) 131 447 4200

Fax: +44 (0) 131 447 4911

www.seebyte.com

# SeeByte

## Points of Contact

Benjamin Privat – SeeTrack CoPilot Project Manager

+44 131-447 4200, benjamin.privat@seebyte.com

## Awareness Made Easy

We demand ever more of the assets and machines we own and operate. We want them to replace us in carrying out hazardous tasks; to work in increasingly harsh and remote locations; and to tell us before they need to be fixed. We also want them to show us what’s going on in their working environment.

To help them do all this we need to make them aware of their mission; aware of their surroundings; and aware of their own condition. SeeByte makes the software that delivers this Awareness.

## Commitment

Our experienced offshore engineering team adhere to strict health and safety procedures and have completed relevant OPITO safety and emergency training and U.K.O.O.A. medical examinations. SeeByte actively pursues policies that promote excellence in safe working practices to ensure there is no harm to people or damage to the environment.

# Hardware Specification

## Front Panel Layout

The position and description of each connector are explained in Figure 1 and Table 1

Figure 1 – SeeTrack CoPilot hardware unit Front Panel Layout

| PART | DESCRIPTION | TYPE |
| --- | --- | --- |
| J1 | On/Off button | Momentary push button |
| J2 | Power Indicator | LED |
| J3 | System Indicator | LED |
| J4 | USB 2.0 port | USB Type A |
| J5 | USB 2.0 port | USB Type A |
| J6 | Drive | DVD-RW/CD-RW |
| J7 | Hard Drive Disk 1 | N/A |
| J8 | Hard Drive Disk 2 | N/A |
| J9 | Reset button | Momentary push button |
| J10 | Fans and filter | N/A |

Table 1– Front Panel Connections

## Rear Panel Layout

The position and description of each connector are explained in Figure 2 and Table 2:

Figure 2 – Rear Panel layout

| PART | DESCRIPTION | TYPE |
| --- | --- | --- |
| J1 | Mains Input 100-240 VAC 47-63Hz | IEC female |
| J2 | Mains Input 100-240 VAC 47-63Hz | IEC female |
| J3 | Mains indicator | LED |
| J4 | Power supply latch |  |
| J5 | Power supply latch |  |
| J6 | PSU indicator | LED |
| J7 | PSU indicator | LED |
| J8 | Mains ON/OFF | ON/OFF Rocker Switch |
| J9 | Mains ON/OFF | ON/OFF Rocker Switch |
| SLOT 1 | E-Stop |  |
| J1.1 | Remote Emergency stop | 2-pin round female |
| SLOT 2 | Display |  |
| J2.1 | VGA | 15-way D female |
| J2.2 | Not used |  |
| J2.3 | Not used |  |
| SLOT 3 | Serial Communications |  |
| J3.1 | Serial ports (up to 4 ports using data cable provided) | 25-way D male |
| SLOT 4 | Spare |  |
| SLOT 5 | IB power |  |
| J5.1 | Interface box power supply | 3-pin round female |
| SLOT 6 | Fan |  |
| SLOT 7 | IO |  |
| J7.1 | Not used |  |
| J7.2 | 10/100 Base-T Ethernet port HAIN UDP | RJ45 |
| J7.3 | 10/100 Base-T Ethernet port Interface Box | RJ45 |
| J7.4 | USB 2.0 port | USB Type A |
| J7.5 | USB 2.0 port | USB Type A |

Table 2 – Rear Panel Connections

Serial Communications

The 4 DB9 rs232 connectors are labelled as follow: (to be defined)

P1/COM1

P2/COM2

P3/COM3

P4/COM4

## Interface box connections

The interface box itself is ROV-specific but allows the interfacing of generic controls such as surge, sway, heave and heading to the SeeTrack CoPilot hardware unit. The interface box connection provides power and switching control lines to allow control to be switched between SeeTrack CoPilot and the default joystick manual control.

Figure 3 – Connecting the ROV Interface Box to the unit

## Interface Box Front Panel Layout

The position and description of each connector are detailed in Figure 5 and Table 3:

Figure 4 – Interface Box Front Panel Connectors

| PART | DESCRIPTION | TYPE |
| --- | --- | --- |
| JI.2 | Power/control lines | 3-pin female |
| JI.3 | 10/100Base-T Ethernet | RJ45 |

Table 3 – Interface Box Front Panel Connector Descriptions

## Interface Box Rear Panel Layout

The position and description of each connector are explained in the following Figure 6 and Table 4.

Figure 5 - Interface Box Rear Panel Connectors

| PART | DESCRIPTION | TYPE |
| --- | --- | --- |
| JI.0 | Joystick connector | DB25 female |
| JI.1 | ROV connector | DB25 male |

Table 4 – Interface Box Rear Panel Connector Descriptions

## DB25 connector wiring layout

Figure 6 - Interface Box DB25 Connectors

The control lines needed are:

- 1 Vertical

- 2 Lateral

- 3 Turn

- 4 Fore/Aft

- 6 GND

The remaining lines are not used and are passed through. This interface box can accept/feedback

- Bipolar joysticks max range +-10Vdc

- Unipolar joysticks max range +10Vdc

## Wiring Diagram