Tethys Motherboard Engineering Change Orders

Chad Kecy

11-03-2009 v1

01-07-2010 v2

22 July 2011 v3	ecm

ECO A/H: Emergency Radio On/Off Initial Conditions Set and Signal Swap with GF_Chan_0

- Lift U94-2 Leg [page 4]

- Lift U94-4 Leg [page 4]

- Lift U51-1 Leg [page 9]

- Solder raised U94-2 Leg to U94-7 (ground) [page 4]

- Solder a wire from V148 [page 1] to U51-1 Leg [page 9]

- Solder a wire from U51-1 pad [page 9] to U94-4 leg [page 4]

ECO B: U84 was populated with the wrong physical part

- Replace U84 with a 74LVC00 (14-SOIC) [page 5 and 13]

ECO C: Multiple resistors populated with wrong values [page 6]

| Resistor | As Installed | Should Be |
| --- | --- | --- |
| R93 | 390 | 33K 5% |
| R94 | 390 | 1K 5% |
| R121 | 390 | 33K 5% |
| R122 | 390 | 1K 5% |
| R123 | 390 | 49.9K 1% |
| R124 | 390 | 10.0K 1% |
| R125 | 390 | 42.2K 1% |
| R126 | 390 | 10.2K 1% |
| R127 | 390 | 20K 5% |
| R128 | 390 | 20K 5% |
| R129 | 390 | 100 5% |
| R130 | 390 | 100 5% |

ECO D: RS-485 Xcvr Timing modifications [page 2]

- Change R4 to 604K 1%

- Lift U83-9 leg

- Lift U83-10 leg

- Solder a wire from U83-10 pad to U83-9 leg

- Solder a wire from U83-10 leg to U83-11

ECO E: Pull up resistors added to Reset signals

- On the backside of the motherboard, add a 10K 5% 1206 0.25W resistor between J30-1 and J26-4. [for /RESIN] [page 1 and 11]

- On the backside of the motherboard, add a 10K 5% 1206 0.25W resistor between C108 and J27-5. [for /SW_RESIN]  [page 1 and 11]

- On the backside of the motherboard, add a wire between U87-12 and J27-5. [page 1 and 11]

- Solder a 10K 5% 0402 resistor across U87-13 and U87-14. [Reset button] [page 1]

ECO F: Change in /RESIN_CPU signal

- Lift U91-2 leg [page 1]

- Lift U91-3 leg [page 1]

- Lift RN9-6 leg [page 12]

- Solder a wire from U91-2 pad [page 1] to V105 [page 12]

- Solder a wire from U91-3 pad [page 1] to RN9-6 pad [page 12]

- Solder a wire from U91-2 leg to U91-7 [page 1]

- Solder a wire from V21 to U91-7 (can combine with step 6). [page 1]

- Cut trace from J1-63 (A63) [page 1]

- Solder a wire from J1-63 to V22 [page 1]

- Solder a wire from U91-8 to U87-5 [page 5]

ECO G: Swap CPU_BURNWIRE_OFF signal with LPC3250_HEARTBEAT signal.

- Cut the trace from J1-65 (row A) [page 1] to U82-13 [page 5].

- Solder a 30AWG wire from J1-65 (row A) [page 1] to V112 [page 11].

- On the backside of the motherboard, run a 30AWG wire frm J27-10 [page 11] to R98 (the side that connects to U82-13). [page 5]

ECO H: see ECO A

ECO I: Changes to the SPI2 bus signals.

- Solder a wire from RN9-7 [page 12] to J27-31 [page 11] (combines the interrupt signals)

- Solder a wire from RN9-1 [page 12] to U89-12 [page 12] (adds a pull up resistor to MOSI)

- Lift U80-1 [page 12]

- Lift U80-2 [page 12]

- Solder a wire from U80-1 leg to U80-7 [page 12]

- Solder a wire from V48 to U80-7 [page 12]

- Solder a wire from U80-1 pad to U80-2 pad [page 12]

ECO J: Add a pull up resistor to UART5 TTL

- Remove U8 [page 2]

- Solder a 10K 5% resistor across U8-9 pad and U8-15 pad

ECO K: Changes to vehicle pressure circuit

- Lift U11-5 leg [page 3]

- Solder a wire from U11-5 leg to U11-4

ECO L: Changes to DVL_TRIGGER_CPU signal

- Lift U93-2 leg [page 10]

- Lift U93-3 leg [page 10]

- Solder a wire from U93-2 leg to U93-7 [page 10]

- Solder a wire from U93-2 pad [page 10] to V144 [page 1]

- Solder a wire from U93-3 pad [page 10] to V145 [page 1]

ECO M: Iridium Emergency Digital Signal

- Remove R52 [page 4]

ECO N: Unused analog input on ADC

- Solder a wire from V45 to U55-13 [page 9]

___________________________________________________________________Rev 3

ECO O:   Remove unused current-sense amplifiers from Instrument Power Supply section, due to leakage currents (BKieft 27 Apr 2010).

- Remove U45, U48, U50, U19   (page 8)

ECO P:    Remove unused current-sense amplifiers from Motherboard Power Supply section (B Kieft 14 May 2010)

- Remove U35, U37, U43, U47

ECO Q:    (B Kieft 14 May 2010)

- Remove TP58 (Batt, page 6) and TP67 (GND, page 7) and replace with Phoenix 2 pin 50 degree screw terminal 3.81 lead space

ECO R:   Mods for NAL Iridium Modem/GPS  (B Kieft 14 May 2010) (superceded by B Kieft, 16 Feb 2011)

- Connect 1k resistor from TP89 (Iridium Power, page 4) to GND (e.g. J21-3 or similar)

ECO S:  Add filtering to ADC input (E Mellinger 11 June 2010)

- Lift lead of U55-6 (or cut trace) (page 9), add 15k 10% 1/10W resistor in series with lead.  Add 0.1 uF/25V ceramic cap (1 uF if size permits) between U55-6 lead and U55-4 or V45 (GND).

ECO T:  Replace tantalum capacitors in high-inrush locations with TVS diodes (B Kieft, 7 Sept 2010).

- Remove C45 and C47 (page 6).  Replace with SMBJ24A TVS diode (Digi-Key SMBJ24A-E3/52GICT-ND

- Change C85 (page 8) to 68uF/20V tantalum (Digi-Key 399-3803-2-ND).

ECO U:  Isolated Burnwire power supply. (E Mellinger, 24 March 2011.  See Drawing “Tethys Isolate Burnwire Mod for off-Motherboard wiring.)

- Remove U28 DC-DC converter (page 5)

- Jumper U28 pad 3 to 6, pad 4 to 7 using AWG #20

- Jumper U28 pad 1 to 8, pad 2 to 9, using AWG #20

- Remove U26 MOSFET (page 5)

- Jumper U26 pad 1 to pad 8 using AWG #20

- Jumper J20 (page 10) pin 8 to pin 6, and pin 3 to pin 5, using AWG #22

- Remove U24 optocoupler (page 5)

ECO V:  Add filtering to SPI CLK at U55 A/D converter (B. Kieft, 17 March 2011)

- Add 2.7k resistor from U55 pin 16 (page 9) to GND (pin 4, pin 13, or V45).

- Add 18pf ceramic cap in parallel with above resistor.

ECO W:  Increase drive current to GF opto U51  (B Kieft, 23 March 2011)

- Lift U54 pin 1  (page 9).

- Remove blue wire from U51 pin 1 (from ECO A/H), leave pin lifted

- Remove other end of blue wire from V148

- Connect U51 pin 1 to pad under U54 pin 1.