10020865rB Four Channel Serial/Power Mux

Board rework directions:

MUXER 5V TO 3.3V Low Power Operation Modification:

1) REPLACE K1, K2 AND K3 DSP2A-L2-DC5V relays with DSP2a-L2-DC3V relays

2) REPLACE U14 LT1129CST-5 with LT1129CST-3.3

MUXER External POR Circuit for U3 Modification:

1) REMOVE J8

2) INSTALL PIN2 OF DS1818-10+ INTO J8.3

3) INSTALL PIN3 OF DS1818-10+ INTO J8.4

4) JUMPER PIN1 OF DS1818-10+ OVER TO SIDE OF R32 THAT MAKES CONNECTION WITH

PIN3 OF U3

MUXER Sample Resistor Change:

1) REPLACE R36 SR10-0.01 with SR10-0.10

MUXER R37 and R38 Change:

1) USE ZERO OHM VALUE

MUXER R23 Change:

1) USE 22AWG Bus Wire with Teflon sheathing

MUXER JP3,4,5,6 Strapping Configuration:

1) JUMPER PINS 2 & 3 FOR JP3, JP4, JP5 and JP6

Luke Coletti

09-May-2013