OUGHTTO:  Remove nets AUXCL, AUXCL_1V8, AUXDA, AUXDA_1V8. These aren’t used on this design.

- DK 11/26/2019: agreed. Those signals are:

  - I2C master serial data, for connecting to external sensors;

  - I2C Master serial clock, for connecting to external sensors

  - According to datasheet, they can be left unconnected.

MAYBE: /IMU_ENABL goes to TLV702’s EN which is high-for-enable positive logic, just take the / off the name of the net

- DK 11/26/2019: Need to check the PIC specs on this pin. What would be a boot-up procedure of PIC, need to make sure with handle IMU interface responsibly during Mother Board start-up/ reboot. We could add inverter into this line, if needed, or use 1.8V regulator with inverted /EN input instead of straight EN

NIT: Please note that the MAX337 limits the SPI frequency to 16Mbps as it may cause confusion in the future.  (See data rate options on the MAX337’s datasheet).

- DK 11/26/2019: Good point. If needs to be faster, we might use another level shifter, preferably with lower power consumption.

KUDOS: I’m glad the SPI lines are on one level shifter and the interrupt lines on the other. This will let me tristate the SPI lines except when they are needed.

- DK 11/26/2019: Having 2 separate tri-state level shifters could indeed be interesting option. Unfortunately we are running out of IOs to be able to control them separately

BUG: Only U2 should have the Tri-State connection (though it still needs the pull up to 1.8V). U3’s Tri-State should not be connected.

- DK 11/26/2019: This only works if INT1_1V8 and INT2_1V8 are active HIGH, and normally sitting low. That way shutting down IMU would keep signals INT1 and INT2 on other side of level shifters low. Otherwise, if INT1_1V8 and INT2_1V8 are normally high, and active low, then shitting down the IMU would send active interrupt signals INT1 and INT2 to PIC

MAYBE: Should the SPI CS be on the other level shifter, so it doesn’t float when the level shifter is tristated?

- DK 11/26/2019: If we shut down the power to IMU, and /IMU_CS signal from PIC on Mother Board is high (not active), then IMU is going to bring down the /IMU_CS signal; or, if current output from PIC is strong enough, it will flow current to IMU internal power thru clamping diodes, preventing complete shotdown/ reboot of IMU. For this reason, I think, we should isolate (all?) the signals that are going to IMU, using Tri-state function of level-shifters.

MAYBE: What happens to the level shifters when IMU ENABLE is not on? The VL is unset, is that safe for the MAX337 chip?

- Good point. We might have to use 2 voltage regulators to 1.8V then

And now questions…

Is this a correct power on process? This would also be the reset process if the system gets into a bad spot and needs the IMU to reset in the field (hopefully not).

- IMU not selected for SPI (IMU_CS high)

- Level shifter tristate on (TriState low)

- IMU selected for SPI (IMU_CS high), talk to chip

- Level shifter tristate off (TriState high)

  - I’m not sure what this does… is MISO high if SPI is configured for it to be?

- Enable LDO, turns on 1.8V (IMU_ENBL high)

According to the ICM-20949: Power up with SCL/SCLK and nCS pins held low is not a supported use case. It will likely set the IMU into I2C mode which I’m not sure how to recover from.

For talking to the SPI during normal running:

- Level shifter tristate off (TriState high), SPI lines are going through

- IMU selected for SPI (IMU_CS high)

- Talk to chip

- IMU not selected for SPI (IMU_CS high)

- Level shifter tristate on (TriState low), SPI lines are not going through

DK 11/26/2019 Reboot procedure meant to be like this:

- Set /Tri-State signal low (active). This will isolate the comm lines to IMU.

- Set /IMU_ENBL high (inactive). This will shot down 1.8V regulator power.

- Wait until 1.8V capacitors discharge (several hundred milliseconds should do it)

  - This should work, UNLESS some of the signals connected to IMU are kept HIGH preventing IMU from going into complete shutdown

- Restore 1.8V power.

- Wait ~100ms until power is stable.

- Activate level-shifter(s), thus connecting comm line from PIC to IMU.