OUGHTTO:  Remove nets AUXCL, AUXCL_1V8, AUXDA, AUXDA_1V8. These aren’t used on this design.

- DK 11/26/2019: agreed. Those signals are:

  - I2C master serial data, for connecting to external sensors;

  - I2C Master serial clock, for connecting to external sensors

  - According to datasheet, they can be left unconnected.

MAYBE: /IMU_ENABL goes to TLV702’s EN which is high-for-enable positive logic, just take the / off the name of the net

- Need to check the PIC specs on this pin.

NIT: Please note that the MAX337 limits the SPI frequency to 16Mbps as it may cause confusion in the future.  (See data rate options on the MAX337’s datasheet).

KUDOS: I’m glad the SPI lines are on one level shifter and the interrupt lines on the other. This will let me tristate the SPI lines except when they are needed.

BUG: Only U2 should have the Tri-State connection (though it still needs the pull up to 1.8V). U3’s Tri-State should not be connected.

MAYBE: Should the SPI CS be on the other level shifter, so it doesn’t float when the level shifter is tristated?

MAYBE: What happens to the level shifters when IMU ENABLE is not on? The VL is unset, is that safe for the MAX337 chip?

And now questions…

Is this a correct power on process? This would also be the reset process if the system gets into a bad spot and needs the IMU to reset in the field (hopefully not).

- IMU not selected for SPI (IMU_CS high)

- Level shifter tristate on (TriState low)

- IMU selected for SPI (IMU_CS high), talk to chip

- Level shifter tristate off (TriState high)

  - I’m not sure what this does… is MISO high if SPI is configured for it to be?

- Enable LDO, turns on 1.8V (IMU_ENBL high)

According to the ICM-20949: Power up with SCL/SCLK and nCS pins held low is not a supported use case. It will likely set the IMU into I2C mode which I’m not sure how to recover from.

For talking to the SPI during normal running:

- Level shifter tristate off (TriState high), SPI lines are going through

- IMU selected for SPI (IMU_CS high)

- Talk to chip

- IMU not selected for SPI (IMU_CS high)

- Level shifter tristate on (TriState low), SPI lines are not going through