Option 1:

| Color | Designation |
| --- | --- |
| Black | Power Common, Return, Ground |
| Brown | Signal Ground, 485/232 Common |
| Red | Primary Logic Power |
| Orange | +12Vdc, +15Vdc |
| Yellow | +48Vdc |
| Green | Chassis / Frame |
| Blue | -5Vdc, -12Vdc, -15Vdc |
| Violet | +24Vdc, +27Vdc |
| Grey | +36Vdc |
| White | Secondary Power Common, Return, Ground |
| Green/White Stripe | Sea Water Ground (18 AWG) |
| White/Brown Stripe | MISC 24-20AWG wire / Misc Signal 1 |
| White/Red Stripe | MISC 24-20AWG wire / Misc Signal 2 / Secondary Logic Power |
| White/Orange Stripe | MISC 24-20AWG wire / Misc Signal 3 |
| White/Yellow Stripe | MISC 24-20AWG wire / Misc Signal 4 |
| White/Green Stripe | MISC 24-20AWG wire / Misc Signal 5 |
| White/Blue Stripe | MISC 24-20AWG wire / Misc Signal 6 |
| White/Violet Stripe | MISC 24-20AWG wire / Misc Signal 7 |
| White/Grey Stripe | MISC 24-20AWG wire / Misc Signal 8 |
| White/Black Stripe | MISC 24-20AWG wire / Misc Signal 9 |
| Black/White Stripe | MISC 24-20AWG wire / Misc Signal 10 |
| Red/Yellow Stripe | Primary High Voltage (18AWG - 22AWG) |
| Red/Green Stripe | Secondary High Voltage (18AWG) |

Communication (Wires twisted with these color combinations will represent the following)

| Color | Designation | Color | Designation |
| --- | --- | --- | --- |
| White/Black Stripe | 485A | White/Green Stripe | Tx |
| White/Brown Stripe | 485B | White/Blue Stripe | Rx |
| Brown | 485 Common | Brown | 232 Common |

Pin to Pin Wiring (Option 2)

| Color | Designation |
| --- | --- |
| Brown | 1 |
| Red | 2 |
| Orange | 3 |
| Yellow | 4 |
| Green | 5 |
| Blue | 6 |
| Violet | 7 |
| Grey | 8 |
| White | 9 |
| Black | 10 |
| White/Brown Stripe | 11 |
| White/Red Stripe | 12 |
| White/Orange Stripe | 13 |
| White/Yellow Stripe | 14 |
| White/Green Stripe | 15 |
| White/Blue Stripe | 16 |
| White/Violet Stripe | 17 |
| White/Grey Stripe | 18 |
| White/Black Stripe | 19 |
| Black/White Stripe | 20 |