Working order for development of SHA probes

- 16S rRNA gene library

  - 27F-1492R bacteria;  21F-958R for Archaea

  - Vector PCR 4.0 for sequencing has both T7 and T3 promotors that flank the insert.  Also has M13f/r primer sites.

- Identify clades of interest and controls for each group

  - Clade in culture (ideal)

  - rRNA containing plasmid to which RNA transcripts can be made

- Design capture probes, ideally within 100 bp of a signal probe (see guidelines)

- Material for testing capture probes.

  - rRNA containing plasmid (see Generating RNA transcript Protocols)

    - Which RNA polymerase promoters are on vector

    - M13 forward and reverse primers include the RNA promoters?

  - Culture

    - Grow cultures, enumerate, and filter on duropore filters for testing (1e6-1e8 cells per filter)

  - Environmental Samples

    - Collect on 25mm filters.  Store in liquid N2 or -80°C until use.

Filter types: (Have been used in ESP)

Duropore:  GV 0.22um GVWP02500

HV 0.45um HVLP02500

Nitrocellulose-based:    GSWP 0.22um GSWP02500

HA 0.45 HAWP02500

Probe Development:

Note:  Only Guidelines---  sometimes probe that do not fit any of the criteria will still work well.….

- Signal probes are Universal (currently have Eub338 and Univ519ab)

- Ideally want captures to be w/in 100 bases of signal probe.  The further you move away you run a great risk of a break in the RNA where the signal and capture are no longer on the same strand.

- Design captures that are >65°C or up to 70-75°C if you can get it.  It allows longer stretches to trim down to optimal Tms.  Many of the BAC array probes are 65-71°C.

- Avoid homodimers and stable stem loop structures

- GT are not mismatches in GuSCN do not rely on those for specificity

- Ideally mismatches are in the middle of the probe.

- We use an Oligo Etc Program (OligoTech) for our final probe Tm determination.  Other calculators are a good start (IDT good one).

Generating RNA transcripts:

- Determine Orientation of Insert in Plasmid to determine the promoter to use to generate the TARGET RNA.

      - Via sequence:  orientation of M13f/r  or T7/T3 promoter to forward or reverse primer used to generate cloned inserts.

        - For Topo 4.0 vector for sequencing

          - M13R, T3---Insert---T7, M13F

          - T3 Kit:  M13F/T7 & Reverse OR M13R/T3 & Forward

          - T7 Kit:  M13F/T7 & Forward OR M13R/T3 & Reverse

      - Via PCR:  Use M13f and Reverse Primer used to generate the insert.  Length amplified determines the orientation.

- Kits:

  - Ampliscribe T7 High Yield Transcription Kit (Epicentre)- T7 only, T3 kit discontinued.  This kit produces 10X the amount of Transcript than the Maxiscript kit.  Do NOT order Durascribe Kit.

  - MaxiScript Kit (Life Technologies)- includes both T7 and T3 enzymes

- Make Freezer stocks of Clones

  - Grow overnight in LB + 7% Glycerol with appropriate Antibiotic

  - Freeze and store in -80°C

- Preparation of Template for Transcription (regardless of Kit selected)

  - Plasmid Miniprep (Qiagen, QiaPrep Spin Miniprep Kit) per manufacturer’s protocol (Any miniprep system should work fine)

  - Amplify Insert with M13F/R (40ul PCR Reaction, Anneal Temp=55°C, 30 cycles.  I use Platinum Taq but any PCR enzyme system should be fine)

  - Load 5ul on 1% Agarose Gel

  - Add 100 ul TE buffer to remainder of PCR reaction

  - Add equal volume of Phenol:Chloroform:1AA ph 8.0 (130ul) , spin 5 minutes, transfer aqueous phase to a new epi tube.

  - Add equal volume of Chloroform:1AA, , spin 5 minutes, transfer aqueous phase to a new epi tube.

  - Add 13ul of 3M NaOAc pH 5.2 and 1 volume of Absolute Ethanol.  Hold at -20°C for at least 20 minutes.  Spin for 30 min.  to pellet DNA.  Decant off liquid

  - Wash with 500ul 70% Ethanol

  - Air dry and resuspend in 15ul water.  Quantify with nanodrop (use only 2 ul)

Protocol Ampliscribe T7 High Yield Kit (follows manufacturer’s instructions)

1.  Add in order: (Bring all components to Room Temperature except T7 enzyme)

20ul Reaction Volume

X ul Template (range:  300 to 1ug max)

Xul Water

2 ul 10X buffer

1.5ul 100mM ATP

1.5ul 100mM CTP

1.5ul 100mM GTP

1.5ul 100mM UTP

2ul  100mM DTT

0.5ul RiboGuard RNAse Inhibitor

2ul Ampliscribe T7 enzyme

2.  Incubate 2hrs at 37°C

3.  Add 1ul RNase Free DNAse, incubate 15min at 37°C

4.  Add 100ul water

5.  Add equal volume of Phenol:Chloroform:1AA ph 8.0 (130ul) , spin 5 minutes, transfer aqueous phase to a new epi tube.

6.  Add equal volume of Chloroform:1AA, , spin 5 minutes, transfer aqueous phase to a new epi tube.

7.  Add equal volume of 5M Ammonium Acetate, incubate on ice 15 minutes.  Centrifuge 15min., Wash with 70% Ethanol

8.  Air Dry.  Resuspend in 50ul Water and determine concentration of RNA with nanodrop. (protocol give ~2ug/ul RNA)

9.  Dilute a working stock of ~ 20ng/ul store at -20°C.  Store the remainder at -80°C

Protocol MaxiScript Kit (Life Technlogies):  Follows Manufacturer’s instructions

1.  20ul Reaction Volume

X ul Template

X ul Water

2ul 10X buffer

1ul ATP

1ul CTP

1ul GTP

1ul UTP

2ul Enzyme (T3 or T7)

2.  Incubate at 37°C for 1 hr

3.  Add 1ul TurboDNase 37°C 15min

4.  Add 1ul 0.5M EDTA

5.  Add 30ul Water

6.  Add 5ul 5M Ammonium Acetate

7.  Add 3 volumes Absolute Ethanol.  Incubate 30 min at -20°C. Spin 15 minutes.  Wash 70% ethanol.

8.  Air dry and resuspend in 50ul water.  Determine concentration of RNA with nanodrop.  (Protocol gave ~100s ng/ul).

9.  Dilute a working stock of ~ 20ng/ul store at -20°C.  Store the remainder at -80°C

SHA reagents for Plate Use: (Order in Bulk or  Development 96 Well Plates)

Bulk order

Buffer E

1 Step Ultra TMB Solution (Thermo Scientific #34038)

1:400 Conjugate

*2M Signal Buffer

*Signal Diluent

*3M GuSCN pH 8.9

*Lysis Diluent pH 8.9

*Probe Assay Wash

*Strepavidin (Thermo Scientific #21125)

Prongs

Evergreen Untreated Sterile 96 well plates (222-8050-KIK)

Development plate without Signal or Capture Probes

You will need * products in bulk to run these plates

Additional supplies:

10% H2S04

300ul 12-channel multi-pipet

300ul barrier pipet tips

Reagent Boats

Upon Arrival of Probes.

- Resuspend probes in water (Sigma, W4502) to 1ug/ul

  - Use the ug/OD constant and OD of yield to determine re-suspension volume

  - Probes remain at 4°C until dried down (within 1-2 days).

- Dry in 25 to 50ug aliquots of probe.  Store at -80°C until use.

Capture Probes for SHA in plate format

- For SHA plate analysis:  resuspend to 100-200ng/ul in TE buffer, quantify on nanodrop.  Store resuspended probes at 4°C.

- Dilute streptavidin (Sigma) in water to an OD of 3.44 (good only for the day)

  - If Strepavidin not yet suspeneded follow manufacturer’s instructions:  add 0.5ml Water and let sit at least 2hrs before first use.  Concentrated stock solution is only good for 1 month.  Store at 4°C

For 20ml of 400ng/ml Capture probe solution (scale appropriately for greater volumes)

10 ml Probe Assay wash + 25ul of Strepavidin (OD 3.44)

10 ml Probe Assay wash + 400ng/ml Probe for 20ml (as determined from Nanodrop readings, i.e add 40ul 200ng/ul probe for 20ml of 400ng/ml)

Combine two solutions, vortex gently and let sit 10 minutes before use.

Store Capture probe solutions at 4°C.

Signal Probe Solution for Bacterial and Archaeal SHA

Resuspend dried down signal probes in TE buffer to ~100ng/ul.  Store resuspended probes at 4°C.

- Signal probe solution for Plate Format

  - Each Signal probe added to a final concentration of 100ng/ml in 0.5M Signal Buffer

- Signal probe solution for use on ESP

  - Each Signal probe added to a final concentration of 100ng/ml in 0.7M Signal Buffer (Also include Alex-Alt at 3.25 ng/ml)

Signal solution is stored at Room Temperature.

SHA Plate Run Protocol:

Each column is a SHA reaction.

Each well of reach row contains 250ul of reagent as specified below

A  Substrate

B  Buffer E

C  Buffer E

D  1:400 Conjugate

E  Buffer E

F  Signal Probe

G  Capture Probe

H  Lysed Sample

Notes:

- In set up, no less than 3 replicates are performed for a Signal-Capture-Lysed Sample combination.

- Typically run a single transcript or sample against a whole plate(s) with a number of different capture probes.

- Pooling environmental sample filters will minimize sample to sample variation.  For one plate you will need 2 filters.

- Cross Reactivity testing using transcripts is done at 50ng/ml lysate.

Protocol for Bacteria and Archaea ONLY!!!!:

(Lysis procedure for HAB and LARV is different)

- Turn on SHA  machine and  heater,  and 85°C water bath

- Let plate come to room temperature

- Add appropriate Signal and Capture probes to wells F and G respectively (see above).

- Lyse Material for analysis.

  - Transcripts:

    - Add transcript to final testing concentration to appropriate volume of 3M GuSCN (1/2 the final volume, see #5)

    - Incubate at 85°C for 5 min.  Vortex at start, 2.5min and end

  - Environmental Samples on Filters

    - Need two filters per plate.

    - If sample was frozen, wait 1 min before adding lysis buffer

    - Add 1ml of 3M GuSCN per filter

    - Incubate for 10 min at 85°C.  Vortex at start, 5min and end

    - Pull lysastes from same filter and push lysate through 0.2um 13mm (SLGVX13NL) or 25mm (SLGV033RS) syringe filter (Millex GV)

- Dilute to 1.5M GuSCN with Lysis diluent

- Apply to 250 ul to each well of Row H on Plate

- Place prong on plate machine.  Support the arm at the free end.

- Add plate

- Push start.

- Upon completion, push start again and remove plate.

- Read in Plate Reader at OD650

- Add 50ul of 10%H2SO4 mix well, wait till all bubbles pop

- Reread on Plate Reader at OD450