Ocean Beach, San Diego Recon 10/1/13

Location of Ocean Beach Pier with respect to the city of San Diego.

View of pier from beach parking lot.

ESP site #2

ESP site #1

Zoom in of Ocean Beach Pier with possible ESP site locations. The ideal deployment site is ESP site #1.

ESP site location #1. Enclosed gated area on pier. Area is secure and locked. There appears to be 110V outlet located with the gated area. This will be checked with the City of San Diego. Approximate footprint of free area is 4 ft by 4 ft (area close to trash can in picture). Notes: shade relief will need to be built here. Elevation above water is ~ 30 to 35 ft. Depth of water needs to be measured. Site is within surf zone and subject to wave and turbulence, complicating pump installation and use. Maybe possible to mount pump of down swell side of piling here.

ESP site location #2. Retail/food establishment and restrooms. Area appears to be secure and locked. High likely hood of power available here.  Utilization of space and power will need to be checked, if this option is utilized. Elevation above water is ~ 30 to 35 ft. Depth of water needs to be measured. Site is still within surf zone and subject to wave and turbulence.

Pier Notes:

Pier is entirely concrete. All plumbing will need to be routed over the wood railing and under the pier to concrete pilings.