- FILE->NEW->DESIGN if required

- Click on Plan View ->Snap to grid -> Circle drawing tool->Outside diameter 4.5 inches

- Line draw from the right-hand (3 o’clock) position of the circle .625-inch length

- Snap line to the tangent point on the circle 1.790 inches

- Select the triangle (ratchet feature) just drawn using the move tool

- Place anchor tool on world origin

- Select create patterns

- Click hold and drag the blue arrow on move tool -> Drag release when more than 3

- Change count to 6

- Trim circle

- Select the Circle draw tool

- Click shift to reference of the center of the circle

- 1.25 inches the X dialog box and 0 in the Y dialog box-> .5 inch for circle diameter

- Select Move -> Select Circle -> Move handle to World origin -> Click create pattern

- Change count to 4

- Sequence -> Orient -> Path ->Select 2D and the face of ratchet wheel

- Right Click and select Export 2D Ord File

Note: Part thickness for extrusion can be changed in the left-hand menu

- When files are exported as 2D ORD files the part thickness must be entered in FlowCut