- FILE -> NEW -> DESIGN -> Open File -> Exercise 10

- Sequence ->Orient -> Extend part faces

  - Not all faces  extend properly

- Delete Contours and Flow Path in structure tree

  - Extend the only the vertical faces (Not the Chamfer faces)

- Use measure tool to identify the vertical curvature

- Creating and Editing cut surfaces

  - COPY and PASTE the Top and Bottom chamfered faces

  - Uncheck the Solid in the structure tree

  - Double click to select the Top and Bottom surface edges

  - COPY and PASTE the surface edges -> Delete the surfaces in structure tree

  - Delete the four curves measuring .176 inches

  - Blend the bottom curves to create a surface

  - Uncheck the curves in the structure tree

  - Select the three top edges of the surface

  - Pull the surface up to a 1-inch height using the angled arrow
(the edges must extend in the same shape as the surface)

  - Check the Solid in the Structure tree

  - Place a plane on the top surface of the part

  - Use the Split body tool -> Select the surface then the plane, click on the surfaces above the plane to delete them

  - Uncheck surface in the structure tree

  - Select the Solid -> COPY and PASTE 
(this is for reference only, and will not be pathed)

  - Right click the new solid and rename -> Uncheck in the Structure tree

  - Check Curves in the structure tree and uncheck everything else

  - Blend the top chamfer curves

  - Repeat steps i-l, for the top curves; pull the surfaces down and place the plane on the bottom face of the solid

  - Check the two created surfaces and the original solid in the structure tree

- Sequencing the part

  - Sequence -> Path -> Use Add/Remove Faces tool located in the Path sub tools to select the surfaces -> Click checkmark

  - Using the Select tool; select left face on the solid -> Right Click -> Select Split Contour

  - Reorder path as needed

  - Modify Leads as needed

  - Export as a 3D Ord File