- FILE -> NEW -> DESIGN -> Plan View -> Open file

*Opposing angles unable to remove the part without relief cuts

- Delete the path from the structure tree

- Orient to a top view

- Design tab -> Sketch Mode->

  - Select the top face of the part

- Line Draw tool: Draw a box around the part

- Uncheck the solid in the structure tree

- Pull tool -> Select the top face of the square -> Start the pull

  - Pull to .375

- Make sure all solids are part of the same component

- Delete: the top and bottom face of the square to create a surface

- Uncheck the surface in the structure tree

- Check the solid in the structure tree

- Sequence -> Orient -> Path using the add/remove faces tool

- Select all angled faces and internal faces of the trapped part -> Click the checkmark

- Uncheck the solid in the structure tree

- Select the surface in the structure tree -> Path -> Checkmark

  - Right Click and select Export 3D Ord File

The new contour will be used to cut the perpendicular faces of the part and act as a relief cut to free the part