- File -> New -> Design

- Plan view

- Rectangle draw tool-> Snap to world origin

- Left click to anchor rectangle enter X: 2 inches, Y: 6 inches

- Three-point rectangle ->Define from center X: 4 inches, Y: 2 inches

- Bend tool ->Left click hold and drag Radius to 1 inch

- Circle draw -> Shift at world origin X: 1 inch, Y: 1-inch press enter

- Input a .5-inch diameter

- Move ->Select circle ->Create pattern

- Drag red handle -> Enter to 4 inches Change count to 4

- Create rounded corner -> Chamfer mode -> Click a corner line and click the opposite line that creates the corner drag mouse and left click when seeing a .5-inch chamfer

- Repeat step 10 for the other three outside corners of the shape

- Offset curve tool-> Double click outer U shape -> Offset .625 inches Repeat and offset .375 inches

- Tangent Arc tool: Cap the slot

- Sequence -> Orient -> Path ->Select 2D and the face of T-bracket

- Right Click and select Export 2D Ord File

Note: Part thickness for extrusion can be changed in the left-hand menu

- When exported as a 2D ORD file thickness must be entered in FlowCut

- .flow2d files can be opened in FlowCut, but not FlowPath