- FILE -> NEW -> DESIGN -> Plan View -> Turn on Snap to grid

- Base shape:

  - Rectangle tool X: 4 inches, Y: 4 inches

- Pull tool:

  - Spin the part slightly

  - Select the surface

  - Pull the surface down to 1 inch

- Rounded Corners:

  - Pull tool: Select a corner edge -> Pull to .5 inches

- Chamfered Corners:

  - Pull tool -> Change general options to Chamfer

  - Select a corner edge -> Pull to .625 inches

- Sequence ->Orient -> Path -> Click green checkmark

- Right Click and select Export 2D Ord File or 3D file

Notes:

## Difference in saving a flow 3D files vs. Flow 3d Ord files

Flow 3D Ord file thickness cannot be changed in FlowCut

Flow 3D files (.flow3d) can be reopened in FlowXpert

Flow 3D Ord Files (.Ord) cannot be reopened in FlowXpert