- FILE -> NEW -> DESIGN -> Plan View -> Turn on Snap to grid

- Outer shape:

  - Rectangle tool X: 4 inches, Y: 4 inches

- Inner feature

  - Circle tool -> Shift from bottom left corner X: 2 inches, Y: 2 inches

  - Enter a 2-inch diameter

- Bevels

  - Pull tool: Select face -> Pull to 1 inch

  - Delete inner surface

  - Select Inner circle top edge and 2 outer connecting edges -> Select chamfer

  - Pull edges in and release the mouse

  - Left click the yellow down arrow

  - Change to distance angle

  - Enter .25 inches @ 45° degrees

- Pathing

  - Sequence ->Orient

  - Extend -> Select top face then bottom face

  - Select all the outer and inner faces -> Checkmark

  - Path -> Checkmark

  - Select -> pick a cut face that isn’t extended

  - Right click -> Split contour

  - Reorder the path as needed

  - Right Click and select Export 3D Ord File

1st

2nd

3rd

**** The order of pathing a part starts from the Bottom (1st) Middle (2nd) and top (3rd)

**** Faces on parts below the top surface must be extended! Or cutting head will crash

Flow 3D files (.flow3d) can be reopened in FlowXpert

Flow 3D Ord Files (.Ord) cannot be reopened in FlowXpert