- FILE -> NEW -> DESIGN -> Plan View -> Turn on Snap to grid

- Base shape:

  - Rectangle tool X: 4 inches, Y: 4 inches

  - Pull the surface down to 1 inch

- Rounded Corners:

  - Pull tool: Select the 4 corner edges -> Pull to .75 inches

- Large inner feature addition:

  - Select Sketch mode or press the letter (K)

  - Select the top surface of the part

  - Circle tool -> Shift from world origin X: 2 inches, Y: 2 inches

  - Enter a 1.25-inch diameter

- Smaller inner feature additions:

  - Circle tool -> Click shift on the center of the circle just drawn

  - 1.25 inches in the X and 0 in the Y -> .25 inches for circle diameter

  - Select Move -> Select Circle -> Move handle to World origin

  - Select create patterns

  - Click hold and drag the blue arrow on move tool -> Drag release when more than 3

  - Change count to 8

- Pulling the new features

  - Select 3D mode or press the letter (D)

  - Select the new features

  - Using pull tool pull the surfaces down through the solid

- Sequence ->Orient -> Path -> Click checkmark

  - Right Click and select Export as a 2D or 3D Ord File

Notes:

## Difference in saving a flow 3D files vs. Flow 3d Ord files

Flow 3D files (.flow3d) can be reopened in FlowXpert

Flow 3D Ord Files (.Ord) cannot be reopened in FlowXpert