- FILE -> NEW -> DESIGN -> Plan View -> Turn on Snap to grid

- Base shape:

  - Rectangle tool X: 4 inches, Y: 4 inches

  - Pull the surface down to 1 inch

  - Circle tool -> Shift from world origin X: 2 inches, Y: 2 inches

  - Enter a 1-inch diameter

  - Pull the surface down to 1 inch

  - Delete unneeded surfaces from the structure tree

- Creating the land

  - DESIGN -> CREATE GROUP -> Plane tool

  - Select the top surface

  - Move tool -> Select the edge of the plane (Will display as neon blue)

  - Move the plane down .75 inches

  - DESIGN -> INTERSECT GROUP -> Split body

  - Select the Solid -> Select the plane Click on white space

  - Pull tool -> Draft Select the Plane -> Select a side face above the plane

  - Enter 45° degrees

  - Uncheck or delete the plane from the structure tree

- Inner Feature Bevel

  - Pull tool -> Draft Select the bottom face of the part

  - Select the inner face of the part

  - Enter (-)20° degrees

- Pathing

  - Sequence ->Orient

  - Extend -> Select top face then bottom face

  - Select the outer faces -> Checkmark

  - Path -> Select the top face -> Checkmark

  - Select a face that is not extended

  - Right click -> Split contour

  - Reorder the path as needed

  - Right Click and select Export 3D Ord File

Notes:

**** Faces on parts below the top surface must be extended! Or cutting head will crash

1st

2nd

3rd

**** The order of pathing a part starts from the Bottom (1st) Middle (2nd) and top (3rd)