- FILE -> NEW -> DESIGN -> Plan View -> Turn on Snap to grid

- Base shape:

  - Rectangle tool X: 4 inches, Y: 4 inches

  - Circle draw -> Shift at world origin X: .5 inches and Y: .5 inches press enter

  - Input a .375 diameter

- Move ->Select circle ->Create pattern

  - Drag red handle -> Enter a 3-inch distance

  - Change count to 5

- Pull tool:

  - Pull the surface down to 1 inch

  - Delete inner surfaces from the structure tree

- Angling the inner features

  - Move tool -> Select an inner face -> Selection tab -> Faces with same area

  - Select Anchor from the move sub tools

  - Place the move tool handle at a circle’s top edge

  - Use the Move tool’s arced arrow to rotate the feature back

  - Enter 30°degrees

- Sequence ->Orient -> Path -> Click checkmark

  - Right Click and select Export 3D Ord File

Notes:

## Difference in saving a flow 3D files vs. Flow 3d Ord files

Flow 3D files (.flow3d) can be reopened in FlowXpert

Flow 3D Ord Files (.Ord) cannot be reopened in FlowXpert