- FILE -> NEW -> DESIGN -> Plan View -> Open file: Exercise 8 Vertical Curvature

- Use the measure tool to identify vertical curvature

  - Measure tool -> If it shows as a spline then there is curvature

Changing the cut face tolerance can help to improve the chance of the system ignoring the vertical curvature

- Fixing part if tolerance changes aren’t enough:

  - Rotate the part to front view

  - Select tool: Select side faces by boxing the inside faces from left to right

    - Make sure not to grab the top or bottom face

  - Delete Side faces by holding Ctrl and pressing delete

    - This feature helps ensure that a side face of a solid isn’t accidentally deleted

- DESIGN -> EDIT GROUP -> Select the blend tool -> Select the top and bottom surfaces -> Checkmark

- Recheck vertical edges for curvature

- Pathing

  - Sequence ->Orient ->Path -> Checkmark

  - Right Click and select Export File