- FILE -> NEW -> DESIGN -> Plan View -> Turn on Snap to grid

- Base shape:

  - Rectangle tool X: 4 inches, Y: 4 inches

  - Pull the surface down to 1 inch

- Creating the drafted face:

  - DESIGN -> ORIENT GROUP -> Change view to Front

  - Plane tool -> Select the front face

  - Move tool: Select the plane edge and move the plane .5 inches into the part

  - DESIGN -> ORIENT GROUP -> Change view to Back

  - Plane tool -> Select the back face

  - Move tool: Select the plane edge and move the plane .5 inches into the part

  - Split body tool: Select the solid and the front plane -> Click on white space

  - Split body tool: Select the solid and the back plane -> Click on white space

  - Pull tool -> Draft -> Select the top surface Select the middle face on the right side -> Enter a (-)45° degrees

  - Uncheck the planes in the structure tree

- Creating a Cut surface

  - Select the drafted feature’s face

  - Right click -> Copy

  - Right click -> Paste

  - Uncheck the solid in the structure tree

  - Pull tool select the two short edges on the surface

  - Select Pull Direction from the Pull tool sub tools

  - Click on the X-axis (red) arrow

  - Pull edges 1.25 inches away from the solid

  - Uncheck the surface in the structure tree

  - Recheck the solid in the structure tree

  - Select the three faces that create the Draft feature

  - DESIGN -> EDIT GROUP Fill tool (F)

  - Fill the drafted feature

- Sequencing the part

  - Sequence -> Path -> Use Add/Remove Faces tool located in the Path sub tools to select the surface -> Click checkmark

  - Using the Select top face tool; Select the top surface -> Click checkmark

  - Using the Select tool; select the face on the solid that was filled in -> Right Click -> Select Detach

  - Reorder part as needed

  - Export as a 3D Ord File