# Rachel Carson Novatel GPS Setup

# Receiver Network Configuration

Hostname: novatel1.rc.mbari.org

IP address: 134.89.32.13/255.255.255.0

The receiver has a web server which can be accessed at its IP address, or at novatel1.rc.mbari.org. This web server displays basic status, and can be used to send setup commands as well. You can also connect via the USB port immediately to the right of the network RJ45 connector.

# Receiver I/O Configuration

Com1 –

- 9600bps/8/n/1

- GGALONG (high resolution format GGA),GPGSA,GPZDA

- For input to Sprint ZDA port NSH 5003 for time synchronization

Com2 –

- 9600bps/8/n/1

- GGALONG (high resolution format GGA),GPGSA,GPZDA

- Spare for expansion

Com3 –

- 9600bps/8/n/1

- GGALONG (high resolution format GGA),GPGSA,GPZDA

- For Input to Ranger 2 and Sprint GPS

ICOM1 (internet com port 1)

- UDP broadcast (134.89.22.255) on port 10001

- GGALONG (high resolution format GGA),GPGSA,GPZDA

- One of these is for winfrog/RCNAV

ICOM2 (internet com port 2)

- UDP broadcast (134.89.22.255) on port 10002

- GGALONG (high resolution format GGA),GPGSA,GPZDA

PPS

- Rising edge, 1ms pulse width

- 1PPS feeds into a 4 channel distribution amplifier to the right of the receiver. Note there is a toggle switch on the right rear of the distribution amplifier, which is used to turn this amplifier on and off.

# Winfrog Setup

A new GPS device has been added to winfrog called novatel1. A new virtual comport has also been added to rcnav, which is bound to UDP port 10001. This GPS device is currently configured with the same offsets as the mx420 GPS antenna (the antennas are close together). The novatel1 GPS is configured as the primary. The MX420 receiver is configured as the secondary.

# Ranger 2 Setup

The Ranger2 navpc is now on the network.

Serial output from the com3 port of the novatel is routed to port NSH/6A. This port is configured as both the ZDA input port for the TOD (time of day) device as well as the GPS input. No configuration has been changed in Ranger 2. The serial cable was moved from the splitter to the novatel GPS receiver

# Sprint Setup

The Sprint PC was briefly on the Rachel Carson network but it has been removed at this date Feb 24, 2021.  A windows update started to install, but at that time it was delayed for 35 days and the machine de-networked.  After de-networking, because this machine is not recommended or supported for windows 10 updates by Sonardyne, the GPS was moved off the network and brought in via a splitter cable from the Ranger 2 GPS/GGA input.  Furthermore, the ZDA time input was also removed from the network and the Novatel software reworked to output serial ZDA out the primary header.  While the header was open, we also added a spare.  So there are now three serial ports tapped out of the Novatel, one spare, one to Ranger 2 GGA and one to ZDA for the Sprint.  The Ranger 2 GGA is split and also goes to the Sprint.

The 1PPS cable has been moved to the distribution amplifier being driven from the novatel. This 1PPS loops through to the NSH as well.

The time synchronization device has been changed from the MOXA-based TCP port (192.168.179.51:5003) to UDP port 10001. This is being fed from the novatel receiver via the icom1 interface.  Based on the first paragraph, this previous statement is no longer true. We did not turn off the UDP broadcast feed, but we changed how sprint receives.  Changing from UDP back to the MOXA based TCP serial ports.

The GPS aiding input has been changed from the MOXA-based TCP port (192.168.179.51:5003) to UDP port 10002. This is being fed from the novatel receiver via the icom2 interface.  Again, based on the first paragraph this statement is no longer true.  The GPS aiding now comes in to the MOXA based TCP ports direct serial from the NOVATEL

Note that the time synchronization and GPS aiding input had been adjusted to feed from the same MOXA port before being changed to UDP. This was done by adjusting the MOXA port configuration to accept multiple TCP connections. It is currently configured to accept up to  4 connections (this is the maximum that the software allows).  Not entirely sure if it is advisable to leave this system configured like this but it is at this time.